EXPERIMENT – 7:

CONTINUOUS-TIME FOURIER TRANSFORM AND SPECTRUM

OBJECTIVES :

(ii) To determine the magnitude and phase spectra of continuous-time signals and analyze their significant spectral components.

(iii) To verify important Fourier transform properties and examine their effects on the corresponding time-domain and frequency-domain representations.

MATLAB CODING :

clc; clear; close all;

%% 7(ii) Determination of Magnitude and Phase Spectra of Continuous-Time Signals

figure;

% Magnitude and Phase Spectrum of a CT Signal

t = -2:0.001:2;

dt = t(2)-t(1);

x = exp(-t.^2).*cos(2*pi*3*t);

N = 4096;

X = fftshift(fft(x,N))*dt;

f = (-N/2:N/2-1)/(N*dt);

magX = abs(X);

phaseX = unwrap(angle(X));

subplot(3,1,1);

plot(t,x); grid on;

xlabel('Time (s)'); ylabel('Amplitude');

title('Continuous-Time Signal');

subplot(3,1,2);

plot(f,magX); xlim([-10 10]); grid on;

xlabel('Frequency (Hz)'); ylabel('|X(f)|');

title('Magnitude Spectrum');

subplot(3,1,3);

plot(f,phaseX); xlim([-10 10]); grid on;

xlabel('Frequency (Hz)'); ylabel('Phase (rad)');

title('Phase Spectrum');

OUTPUT :

To determine the magnitude and phase spectra of continuous-time signals and analyze their significant spectral components.

To verify important Fourier transform properties and examine their effects on the corresponding time-domain and frequency-domain representations.

RESULT :

The continuous-time Fourier transform of representative signals was successfully obtained using FFT-based approximation, and their magnitude and phase spectra were analyzed to identify significant frequency components. The time-shift property was verified by comparing the computed and theoretical Fourier transforms, confirming that time shifting preserves magnitude while introducing a frequency-dependent phase shift. The spectral components and occupied bandwidth of a composite communication signal were also determined, demonstrating the importance of frequency-domain analysis in communication-system design.