EXPERIMENT – 8:

LAPLACE TRANSFORM, INVERSE TRANSFORM, ROC AND STABILITY

OBJECTIVES :

(i) To determine the inverse Laplace transform of a given frequency-domain representation and reconstruct the corresponding time-domain signal.

(ii) To determine the Region of Convergence (ROC) of Laplace transforms and analyze its significance in characterizing continuous-time signals and systems.

MATLAB CODING :

clc; clear; close all;

%% 8(ii) Determination of Inverse Laplace Transform for Signal Reconstruction

figure;

% Inverse Laplace Transform

% Requires Symbolic Math Toolbox

syms s t

X = 1/((s+2)*(s+3));

x = ilaplace(X,s,t);

disp('Inverse Laplace Transform x(t) =');

pretty(x);

tt = 0:0.001:5;

xx = double(subs(x,t,tt));

plot(tt,xx,'LineWidth',1.2);

grid on;

xlabel('Time (s)');

ylabel('Amplitude');

title('Signal Reconstructed using Inverse Laplace Transform');

MATLAB CODING :

clc; clear; close all;

%% 8(iii ) Analysis of Region of Convergence (ROC) and Its Significance in System

Analysis

figure;

% ROC Analysis for a Rational Laplace Transform

% Requires Symbolic Math Toolbox

% X(s) = 1/((s+2)(s+4))

% Poles are at s = -2 and s = -4.

p = [-2 -4];

plot(real(p),imag(p),'rx','MarkerSize',10,'LineWidth',2);

hold on;

xline(-2,'--'); xline(-4,'--');

grid on;

xlabel('Real(s)'); ylabel('Imag(s)');

title('Poles and Candidate Regions of Convergence');

legend('Poles','Pole at -2','Pole at -4');

fprintf('Poles are at s = -2 and s = -4.\n');

fprintf('For a right-sided signal, ROC: Re(s) > -2.\n');

fprintf('For a left-sided signal, ROC: Re(s) < -4.\n');

fprintf('For a two-sided signal, ROC: -4 < Re(s) < -2.\n');

OUTPUT :

To determine the inverse Laplace transform of a given frequency-domain representation and reconstruct the corresponding time-domain signal.

To determine the Region of Convergence (ROC) of Laplace transforms and analyze its significance in characterizing continuous-time signals and systems.

RESULT :

Thus, the inverse Laplace transform was determined to reconstruct the corresponding time-domain signal. The Region of Convergence (ROC) was identified, and its significance in analyzing continuous-time signals and system stability was studied successfully.