EXPERIMENT – 6

PARSEVAL'S RELATION AND HARMONIC SPECTRUM ANALYSIS

OBJECTIVE :

(i) To verify Parseval’s relation for continuous-time periodic signals and analyze the

relationship between time-domain and frequency-domain power.

(ii) To compare the total power obtained in the time domain with the harmonic power

distribution in the frequency domain.

MATLAB CODING :

clc; clear; close all;

%% 6(i) Verification of Parseval’s Relation for Periodic Signals

figure;

% Verification of Parseval's Relation

f0 = 1;

T = 1/f0;

t = linspace(0,T,4000);

x = max(sin(2*pi*f0*t),0);

% Time-domain average power

P_time = (1/T)*trapz(t,abs(x).^2);

% Fourier-series coefficients

N = 20;

a0 = (2/T)*trapz(t,x);

an = zeros(1,N); bn = zeros(1,N);

for k = 1:N

an(k) = (2/T)*trapz(t,x.*cos(2*pi*k*f0*t));

bn(k) = (2/T)*trapz(t,x.*sin(2*pi*k*f0*t));

end

% Frequency-domain power using Parseval's relation

P_freq = a0^2/4 + 0.5*sum(an.^2 + bn.^2);

fprintf('Time-domain power = %.6f\n',P_time);

fprintf('Frequency-domain power = %.6f\n',P_freq);

fprintf('Absolute error = %.6e\n',abs(P_time-P_freq));

bar([P_time P_freq]);

set(gca,'XTickLabel',{'Time Domain','Frequency Domain'});

ylabel('Average Power'); grid on;

title('Verification of Parseval''s Relation');

MATLAB CODING :

clc; clear; close all;

%% 6(iv) Comparison of Time-Domain Power with Frequency-Domain Harmonic Power

figure;

% Time-Domain and Frequency-Domain Power Comparison

f0 = 1; T = 1/f0;

t = linspace(0,T,4000);

x = max(sin(2*pi*f0*t),0);

P_time = (1/T)*trapz(t,x.^2);

N = 20;

a0 = (2/T)*trapz(t,x);

an = zeros(1,N); bn = zeros(1,N);

for k = 1:N

an(k) = (2/T)*trapz(t,x.*cos(2*pi*k*f0*t));

bn(k) = (2/T)*trapz(t,x.*sin(2*pi*k*f0*t));

end

P_dc = a0^2/4;

P_harmonics = 0.5*sum(an.^2+bn.^2);

P_frequency = P_dc + P_harmonics;

fprintf('Time-domain power = %.6f\n',P_time);

fprintf('DC power = %.6f\n',P_dc);

fprintf('Harmonic power = %.6f\n',P_harmonics);

fprintf('Frequency-domain power = %.6f\n',P_frequency);

bar([P_time P_frequency]);

set(gca,'XTickLabel',{'Time Domain','Frequency Domain'});

ylabel('Average Power'); grid on;

title('Time-Domain vs Frequency-Domain Power');

OUTPUT :

To verify Parseval’s relation for continuous-time periodic signals and analyze the

relationship between time-domain and frequency-domain power.

To compare the total power obtained in the time domain with the harmonic power

distribution in the frequency domain.

RESULT :

The experiment successfully verified Parseval’s relation for continuous-time periodic signals by demonstrating the equivalence of power calculated in the time and frequency domains. The distribution of power among the fundamental and harmonic components was analyzed through the harmonic spectrum, and the total time-domain power was found to be consistent with the sum of harmonic powers in the frequency domain.